Based on the information in the sources, you can automate the segmentation of the markers in your 3000 ocean images by adopting a strategy similar to the fully automatic stage of SAM's data engine, which was used to generate the large SA-1B dataset. This involves using a regular grid of point prompts across your images. Here's a detailed approach:

- Generate a Regular Grid of Points: Instead of manually providing point prompts, you will programmatically create a dense grid of points that systematically covers each of your 3000 ocean images. The density of this grid (e.g., a 16x16 or 32x32 grid of points) will determine how many prompts are sent to SAM for each image.

- Prompt SAM with the Grid Points: For each point in the generated grid for each of your 3000 images, you will use it as a foreground point prompt to the Segment Anything Model. SAM is designed to produce a valid segmentation mask for any given prompt. For a single point prompt, SAM can predict multiple masks to account for potential ambiguity.

- Filter the Generated Masks: After prompting SAM with the grid of points for each image, you will obtain a large set of candidate masks. You will need to filter these masks to isolate those that are likely to correspond to your markers. The data engine employed several filtering techniques that you can adapt:

  - Confidence Scoring: SAM predicts a confidence score (estimated IoU) for each generated mask. You can set a threshold and discard masks with confidence scores below this threshold.

  - Stability Filtering: The data engine checked for mask stability by comparing binary masks resulting from slightly different thresholding of the probability map. A mask was considered stable if the IoU between masks thresholded at 0.5 - δ and 0.5 + δ was above a certain level (e.g., 95%). You can implement a similar stability check to filter out unreliable masks.

  - Size Filtering: Markers in your ocean images likely have a characteristic size range. You can filter out masks based on their area (number of pixels). Discard masks that are significantly smaller or larger than the expected size of your markers.

  - Non-Maximal Suppression (NMS): Since multiple nearby grid points might generate masks for the same marker, you can use Non-Maximal Suppression (NMS) to remove redundant or highly overlapping masks. This involves ranking the masks (e.g., by their confidence score) and discarding those that have a high overlap (above a certain IoU threshold, e.g., 0.7) with a higher-ranked mask.

- Optional Post-Processing: Depending on the results of the filtering and the specific characteristics of your markers (e.g., shape, appearance), you might consider additional post-processing steps to refine the segmentation masks further.

By implementing this automated approach using a grid of point prompts and subsequent filtering, you can process your 3000 ocean images without manually clicking on each marker, similar to how the SA-1B dataset, containing over 1 billion masks from 11 million images, was created using a fully automatic process. The fully automatic stage in the data engine involved prompting with a 32x32 regular grid of points on the full image and additional zoomed-in crops.

Keep in mind that the optimal grid density and filtering thresholds might require some experimentation to achieve the best results for your specific type of markers and ocean images.